
Specification parameters
	Order
	Parameter
	Specification parameters

	one
	Comparison mode
	1：N

	two
	Collection head type
	Self learning capacitive press fingerprint head

	three
	Acquisition head resolution
	508dpi

	four
	Rejection rate
	＜0.1%

	five
	Fake admission rate
	＜0.001％

	six
	Fingerprint collection range
	±180°

	seven
	Card Type
	M1

	eight
	user capacity
	Total capacity of 250 sets: up to 100 fingerprints, up to 100 passwords, up to 50 faces, and up to 100 cards. Administrator ID range 000-009, regular user ID range 010-249

	nine
	Operation records
	Scroll 600 strip

	ten
	Battery life
	Number of door openings greater than or equal 3000 times

	eleven
	working voltage
	7.0V~8.4V, low voltage alarm voltage 6.8 ± 0.2V

	twelve
	Dynamic power consumption
	less than or equal 1A (no motor action, no playback sound

	thirteen
	Static power consumption
	less than or equal 200uA (average power consumption

	
fourteen
	Verification time
	less than or equal 3s, (including the entire process of fingerprint collection, comparison, and unlocking signal output

	fifteen
	keyboard type
	Touch sensing

	sixteen
	(OLED display screen
	0.96” OLED 

	seventeen
	External emergency power supply for display screen
	5V1A  Micro USB

	eighteen
	Antistatic ability
	≥8KV (contact)  ≥15KV(Non-contact type)

	nineteen
	storage temperature
	－25℃ ～ 80℃

	twenty
	operation temperature
	－25℃ ～ 70℃

	twenty-one
	ambient humidity
	15%RH～ 93%RH

	twenty-two
	Wi-Fi frequency band
	2400 MHz-2483.5 MHz


Setting Description
1.initialization

Long press the settings button for 5 seconds, and the voice will broadcast: "Factory reset, NO(*) YES(#)." Press the # key to confirm, and the system will restart to its initial state.

In the initial state, any face, fingerprint, password, or card can unlock the device. Performing a factory reset operation cannot clear the set door-opening direction or torque parameters.

2.Low battery warning 

If the system's battery level is less than 25%, the system will voice prompt "Low power" ,The battery can still be unlocked about 50 times, but please charge it in a timely manner (or replace the battery).
3.External USB emergency power supply
Using USB emergency power supply, it takes approximately 30 seconds to unlock.

4.Illegal user alarm

Failed to unlock after 3 to 5 consecutive inputs, with voice message: 'Illegal user, illegal intrusion, owner notified'. After the 5th broadcast, the system is locked for 90 seconds, during which time no user action is taken. Unlock after 90 seconds.
5.Virtual password

When using a password to unlock, any number can be added before and after the correct password, which can increase the security of the digital password. For example, if the password is "123456", you can enter "426123456745" to unlock.
Note: The system can only recognize the first 16 digits

6.version number

Enter 999 # to read the system version number.

Function and operation
User settings add administrator password for the first time




1.Add User

· Face: The voice will say "Enroll face." The user needs to position their face parallel to the camera at a distance of 30-80cm. Follow the voice prompts to complete the continuous face recording.

· Fingerprint: The user places their finger on the fingerprint sensor, and the voice prompts "Press again." The user then places the same finger on the sensor to complete the continuous fingerprint recording.

· Password: The user enters a 6 to 8-digit password and confirms it by pressing the # key.

· Card: The user brings the card close to the IC card sensing area on the touch keypad, and the system automatically reads and saves the card information.

Note: Number of management users: numbered 000-009, with a total of 10 digits; Number of ordinary users: numbers 010-249, a total of 240 characters.
2.Delete user
（1）Verify Delete

Verification method:

·Fingerprint: Place the finger to be deleted on the fingerprint sensor.
·Password: Enter the password to be delete.

·Card: The IC card to be deleted is located near the sensing area of the touch keyboard.

If the user exists in the system, the voice broadcast will read: "Success"; If the user does not exist, the voice broadcast will say  "Failure".

（2）Delete by ID

3.System settings
（1）datetime settings


（2）Installation settings
①Door opening direction setting



Note: The direction of opening the door is in the direction of opening the lock outside the door.
②Auto-lock

Note: Delayed locking refers to setting a fixed anti lock time. Users can choose the locking time: 10S, 15S, 20S, 30S, 60S, 120S.

③Lock settings
-Tamper Alarm



Note: After activating the anti tamper alarm, if the door lock is tampered with, the system will issue an alarm prompt: "Illegal user, illegal intrusion, owner notified". If the device is connected to the network, it will push alarm information to the phone.

-Motor Torque

Note: Motor torque is specific to the strength of the motor's torque.

-Lock back time


Note: The rebound time refers to the rebound time of the locking tongue, and the set time range is between 2s and 9s.

-Dwell Alert


Note: Within 1 minute, if a human body is detected for a total of 20 seconds, a stay alarm will be triggered. The pushed network devices will not receive sound and light alarms by default.

④Face recognition settings

-Lock back time

 

Note: 1. Classification of human sensory function:

·Human body sensing [off]: The intelligent door lock will not engage in human body sensing in standby mode.

·Human body sensing [intelligence]: The intelligent door lock will not engage in human body sensing when the door is opened.

·Human body sensing [open]: In any state, the intelligent door lock will undergo human body sensing.

2.If there are 10 consecutive awakenings and no facial recognition is detected, human sensing will be turned off by default for 5 minutes.

-Detection distance


Note:Detection distance[Near]：About 30～50cm；Detection distance[Medium]：50～70cm；Detection distance[Far]：70～80cm.

-Recognition prompt


Note: When the recognition prompt is turned on and the human body senses awakening, the voice prompt is: "Please capture a frontal face"

（3）Function settings

①Network configuration


(Please refer to the appendix of the manual for detailed operation of networked products).

Note: The distribution function only supports one mobile phone for distribution. If you enter the networking configuration again, the last distribution data will be automatically cleared. Users should be cautious when operating.

②Volume


③Dual authentication


Note:Dual authenticationOnly for ordinary key unlocking, two different ordinary user keys must be verified to successfully unlock, and administrator users can directly unlock.

（4）Factory reset

Note: After restoring the actory reset, all user data will be cleared. Please operate with caution.

4.information inquiry
（1）User Count

（2）Read Records
-Inquiry by order

Note: Sequential query starts with the latest information and goes down. Press the 8th key to search for the next item and the 2nd key to search for the previous item.
-Inquiry by time


Appendix 1：
"SmartLockApp" Wechat Applet

1.Overview
The Wechat applet can be used to generate time-limited passwords. These time-limited passwords are generated by a secondary encryption to the unlocking password and time. It can be used even if the smart lock is not connected to the network.

2.Preparation

The User ID 000 has already been added to the fingerprint lock, and the system time has been set correctly.

3.Adding Wechat Applet

Open your Wechat and scan the QR code below to add the WeChat applet "SmartLockApp"
[image: image1.png]



4.Entering the Unlocking Password

Enter the unlocking password of User 000 on the home page of the applet, and click the OK button to confirm and enter the "time-limited password" generation interface 
[image: image2.jpg]il FEEE = £F12:52 © 99% -
SmartLockApp o ®





5.Generating Time-Limited Password

Select the password mode and click the Generate Password button

         [image: image3.jpg]il FE R E#12:53

< SmartLockApp

or Renting Nanny Manager

Effective time 10 Minute >

Note: Please confirm that the time on the lock
has been calibrated before use.



 [image: image4.jpg]E#12:53

SmartLockApp
‘ Visitor Nanny ‘ Manager ‘
Start Time 2019Y5M26D0H >
End Time 2019Y5M26D1H >

Note: The password should be used at least
once within 24 hours after the effective time,
otherwise it will invalid.




[image: image5.jpg]all FEEE = EF12:53

< SmartLockApp

‘ Visitor

Round Mode Everyday >

Start Time 0 Hour >

End Time 1 Hour >

Note: The password should be used at least
once within 24 hours after the effective time,
otherwise it will invalid



 [image: image6.jpg]all FEEE = EF12:53

< SmartLockApp
‘ or Renting Nanny I!m
Type Delete all passwords >

Note: the password is valid for 24 hours, which
is used to invalidation the password used on the
lock.




Visitor mode
In visitor mode, users can generate time-limited passwords for limited 1 to 59 minutes, counting from the current time, which will become invalid after the time limited.

Renting mode
In short-term lease mode, users can generate valid passwords for any period of time. The validity time must be longer than the validity time and the current time. The maximum time range cannot exceed 60 days. The generated temporary password must be entered within 24 hours after the password takes effect. Otherwise, the password becomes invalid.
Nanny mode
In nanny mode, users can set the allowable time periods for unlocking every day. The time losing efficacy shall be later than the time taking effect. The generated password needs to be input within 24 hours, otherwise it will be invalid.

Password Management
The password management function can be used to delete the time-limited passwords that are already in effect ona lock. However, it is not possible to delete the time-limited password that takes effect within 24 hours, administrator user's password, or standard user's password. If you need to invalidate all time-limited passwords, you need to delete User 000 on the lock first, and then add it again.

appendix
APP Operation

App Store search [Smart Life] Download and sign in.

 1、Add device
After entering the APP, click "+" in the upper right corner of the home page to add the device, and enter the "Add Device" page;
1）Two-dimensional code distribution network

- Select [Camera&lock] and click "Video lock (Wi-Fi)" to jump to the next page.

- "Reset the device" interface, click Next, check the Scan the OR code with door lock camera,click [Next].

- Select WIFI and enter password, click [Next].

- After entering the "Reset the devica" page, keep waiting, go to the smart door lock operation, let it enter the network mode. 

- Point the QR code of the mobile phone at the camera. When the lock end plays a "stomp" prompt tone, click "I Hear a prompt" to enter the distribution state.
  [image: image7.png]ting

Large
Applia

Kitch
Applia

Exerc
ea

Came

lth

ra &

Lock

Gateway

Ener

Add Device

Smart
Doorbell
(Duat Band)

MR

Lock
(Wi-Fi)

Lock
(NB-ioT)

Floodight

Camera

ovR

Lock
(2igbee)

Video lock

2]
HH

Base Station

L}

Bird Feeder
wif)

Lock
(8Le)



  [image: image8.png]X GR Code for Camera &

Reset the device

Scan the QR code with the door lock
camera

@  Scen the OR code with the door lock
camera

ook —




[image: image9.png]X

Select 2.4 GHz Wi-Fi Network and
enter password.

‘Common router settng method

e 240m .

T 8T-RD



 .  [image: image10.png]e B asw@

Please scan the QR code from 15 to 20 cm away

rd 3 Prom

No Prompts



[image: image11.png]Connecting Device
Power on the device.

01:59

Scan
devices



[image: image12.png]X Add Device

s) added successfully

SmartLock




      

2）WIFI distribution network
- Select [Camera&lock] and click "Lock (Wi-Fi)" to jump to the next page.
- Select WIFI and enter password, click [Next].
- "Reset the Device" interface, click Next, select 【EZ Mode 】.
- After entering the "Add device" page, keep waiting, go to the smart door lock operation, let it enter the network mode. 

- If the power distribution fails, you can select "Switch the power distribution mode" or select the hotspot mode. Connect to the Wi-Fi hotspot starting with SmartLife, and configure the network according to the instructions of the mobile phone.
Note: Some device hotspot names may be user-defined
 [image: image13.png]< Add Device e =
Lighting Came
Applian (Wi-Fi) (24GH2856 (BLE)
N - (4G) Camera Doorbell
Exercise & '
Camera & Doorbell Camera
Lock: (Dual Band)
contre . .
& Lock Lock




   [image: image14.png]X

Select 2.4 GHz Wi-Fi Network and
enter password.

‘Common router settng method

e 240m .

T 8T-RD



 .[image: image15.png]Select the status of the indicator light or
hear the beep:




  [image: image16.png]Connecting Device
Power on the device.

01:59

Scan
devices



[image: image17.png]X Add Device

s) added successfully

SmartLock




 [image: image18.png]Failed to add the device

Check demerit points and retry

Check if the device has been reset and

the indicator is blinking quickly.
Check if itis 2.4 GHz Wi-Fi
Verify the Wi-Fi password
More device-pairing FAQs

Report Issue



     [image: image19.png]X

Connect your mobile phone to
the device's hotspot

@ Connect your phone to the hotspot shown
below:

VLAN ©
v SmartLife-XXXX

v SL-XXXX

@ Go back and add devices.

Go to Connect




2、Remote unlocking

- Visitors ring the doorbell, the user's mobile phone can receive a remote unlock request.

- Click the adaptive broadcast button to talk to the visitor.

- Slide the release button to the right for remote unlocking.
                [image: image20.png]The lock is begging to open

e -




 [image: image21.png]The lock is begging to open ok

11-15-2023 16:10:44

A
= 0 - -

muste  Voice Click screen —

< -




    

3、Record query

Messages such as doorbell push, illegal user, low battery, and unlocking can be queried in the log.
    [image: image22.png]SmartLock

© Security Protection for 1 Days

Q  15:27 Password Unlock

Member Management
TFamily Member(s)

e Temporary Password
Online/Offline Password

° Smart Linkage

N



     [image: image23.png]Aers Operston Logs
password0 Unlock
1746

The unlocking method is not associated
member in the app.

Associ )

Tuya provide you it e clowd tcage forup 0 30 video events witin
Sdays. You can chase cloud storage esoutce of Toyo o ncresse e
Stoage duaton ad capocy





4、Member management
- Click [Member Management] to enter the member management page.

- Click [+] in the upper right corner to select other members.

- Enter the name of the member to be bound.
-On the Member information page, click Manage Unlock Mode.

-Select the user number to be bound and click OK
Note: Account number - User code generated when the door lock is added.
 [image: image24.png]< Member Management +
An unlocking method is not associated with a member. Associ.

Admin Member

o ) Admin
P J



  [image: image25.png]Family Member

Other Member




[image: image26.png]< Member Management  Save

Name Any




[image: image27.png]< User Details

Any

M FingerPrint

© Password

= Card

® Face

=

To delete the member, go to Me > Home Management,




[image: image28.jpg]Select Unlocking Method

An unlocking method can be associated with only one
Code(1)
PasswordO

Card(1)

Card10




[image: image29.jpg]< User Details

Any

M FingerPrint
Password

B Card

card10

® Face

To delete the member, go to Me > Home Management.

Associate Unlocking Methods




[image: image30.png]<« Logs

Alerts Operation Logs

Today

Any Unlock with card10
17:49

Any Unlock with card10
17:49

password0 Unlock
17:46

The unlocking method is not associated with a
member in the app.

Tuya provides you with free cloud storage for up to 30 video events within
3days. You can purchase cloud storage resources of Tuya to increase the
storage duration and capacity




[image: image31.png]< User Details

Any

M FingerPrint

© Password

= Card

® Face

To delete the member, go to Me > Home Management,





5、Temporary password
                                 [image: image32.png]< Temporary Password

Online Password

© rime-Limited Password

Offtine Password

@ one-Time Passviord

© rime-timited password




1）One-time password

Select a one-time password and click Add or the + sign in the top right corner; Click to obtain the password, the system automatically generates a set of ten-digit password, click to complete.

The password must be used within 6 hours after it is generated. The password can only be used once within the valid period.
        [image: image33.png]s

=)

No valid Password.

Add

View Invald Password >



  [image: image34.png]Create One-Time Password

H KK R K K K K K K

" password s vald for & hours and ca




  [image: image35.png]The one-time password is obtained.

clearing code immediately. The clear ot s

2671847259

Copy
Password Name

Message Notification





2）Unlimited password

Select an unlimited password and click Add or the + sign in the top right corner; Select effective time and implementation time; Click to obtain the password, the system automatically generates a set of ten-digit password, click to complete.
    [image: image36.png]s

=)

No valid Password.

Add

View Invald Password >



 [image: image37.png]<

Create Time-Limited Password

Save the clearing code immeditely The clearing code s not displayedin
the 3pp.

Effective Time 11-17-2023 13:00 >

Expiration Time >

You need to select the effectve and expiratio time before you can obiain the




  [image: image38.png]<

The time-limited password is obtained.

Save the clearing code immedistely. The clear

P

s not dplayed in

2937741325

Copy

Password Name

Effective Time T1-17-2023 13:00

Expiration Time 11-17-2023 14:00

Message Notification

ie time-limited password ot least o
word takes effect. Otherwise, t
passvord mmedistely. The password

ord becomes invalic
s ot displayed after tis





Wake up touch panel








Voice announcement:“Please enter a 6 to 8 digit password”(enter password)








After successful addition, voice broadcast::“User 000Add Success”





Press the *+# keys








After successful addition, voice broadcast: "User * * *, Add Success” can be added continuously








Please enter the password, fingerprint, or card you want to add








1.Add Admin


2.Add user





1.Add User





After successful verification, voice broadcast: "User * * *, Delete Success”








Voice announcement: "Please verify the user to be deleted"








1.Verify Delete





2.delete user





After successful deletion, voice broadcast:"User * * *, Delete Success"








Voice announcement


:Please enter the user ID to be deleted





2.delete  by ID





2.delete user





After successful networking, voice broadcast: "Success"








Voice announcement:"Please enter the datetime" (input time format is: year month hour minute)





1.datetime settings





3.System settings





1.Opening direction【L】





2.Installation settings





3.System settings





2.Delayed lock【N】





2.Installation settings





3.System settings





2.Installation settings





3.System settings





3.Lock settings





1.Tamper Alarm【N】





2.Motor Torque【M】





3.Lock settings





2.Installation settings





3.System settings





2.Installation settings





3.Lock settings





3.Lock back time【2s】





3.System settings





2.Installation settings





3.Lock settings





4.Dwell Alert【20s】





3.System settings





3.System settings





2.Installation settings





4.Face recognition settings





1.Auto Detect【Y】





2Detection distance【M】





2.Installation settings





4.Face recognition 





3.System settings





2.Installation settings





3.ystem settings





4.Face recognition 





3.Recognition prompt【Y】





After successful networking, voice broadcast: "Success"





1、Network configuration





3.Installation settings





3.System settings





3.Installation settings





2、Volume【H】





3.System settings





3.System settings





3.Installation settings





3、Dual authentication【N】





Confirm the restoration of factory reset, voice broadcast: "Clear success", system restart





4、Factory reset





3.Installation settings





3.System settings





Voice announcement: 


           “Admin***


User***”





1.User Count





4.information inquiry





2.ecord inquiry





1、inquiry by order





4.information inquiry





Enter the time you want to query (time is year, month, day)





2.ecord inquiry





2、inquiry by time





4.information inquiry









